 Chapter 7082    QUICK REFERENCE GUIDE   - LUG’s
For complete official language visit www.revisor.leg.state.mn.us



vers 22.2
Chp 7082 regulates LUG responsibilities.

7082.0040
Various levels of Administration Responsibility.



The MPCA is responsible for providing administrative procedures for permitting and inspecting 


SSTS’s, and to verify that local administration is sufficient to ensure compliance.


Counties must adopt and enforce Chapters 7080-81 throughout the county except in those 


towns and cities which have adopted there own ordinances that are as strict at the counties.



Counties must send written invitations to all towns and cities requesting their input to the 


county-wide ordinance development.


To adequately enforce programs, LUG’s must have at least one certified inspector employed by 


the LUG or sub-contract the work to a licensed inspection business.  The LUG must also 



employ at least one person who has received training on administering local SSTS programs.


LUG’s must provide an annual report to the commissioner no later than Feb 1 to include:




-The name and address of the program administrator, all qualified employees and 





contracted businesses.




-The number of permits issued in the different categories as listed in sub 5 item B.



-The total # of full-time, seasonal, cluster, and ‘other establishments’ in the jurisdiction.




-The estimated % of compliant systems in the jurisdiction.




-The # of tanks installed by each installer or homeowner.




-The # of systems regulated by an operating permit.




-for counties, to list the towns and cities that have their own local ordinances.




-and a comment on problem areas in the SSTS administration.

7082.0050
Requirements for ordinances.


Counties must choose and implement either a conventional program, a performance program, or 


alternative local standards.  Towns and cities must adopt the option used by the county and be as 


strict as county ordinance, however, they need not adopt a performance program if they are not 


capable of fulfilling its requirements.


Conventional program to include:  



-ISTS and MSTS “technical” standards as found in chapters 7080 and 7081.



-“Administrative” functions as found in 7082.0100 subp 1,2,3 &5 and 7082.0300-0700 



   which include provisions requiring:





 -the upgrade of a failing system.





 -the upgrade of an IPHT within 10 months or less.





 -1996 or older, non SWF systems, to have 2’+ vertical separation.




 -a list of technical and administrative differences between 7080-81 and the LUG.





 -that all SSTS work be performed by a properly licensed person.





 -that SSTS’s no longer being used, are to be properly abandoned. 





 -acknowledgement of variances allowed, and the procedures required.




 -provisions for design review, permitting, construction review & system mgmt.





 -that newly created lots have 2+  Type I (minus gray water) soil  treatment areas.




 -conditions needed to allow holding tanks, along with monitoring requirements.





 -a provision prohibiting surface discharge without an NPDES permit.





 -a provision specifying the use of systems in a floodplain. 





 -that a mgmt plan be submitted before issuing a new or replacement permit.




 -that operating permits are required for  Type IV, V & MSTS systems.





 -for systems not under a mgmt plan already, that a tank inspection/pumping occur





at least every 3 years.





 -class V injection wells submit inventory information to EPA and disclose 





existence at property transfer.





 -procedures on how to resolve compliance inspection disputes.





 -what level of LUG approval is necessary to repair SSTS’s.




 -what methods are allowed to determine SLR (pit and perk).





-provision that all sewage be treated in a municipal plant or per 7080-83.





 -if a reduction of 3’ vertical separation is allowed, that it not exceed 15%.





-provision that it is ok to be more restrictive than 7080-83.





 -also to include sections 7082.0300 thru 7082.0700.


Performance program to include:  




All the Technical and Administrative requirements of the Conventional program and 



a comprehensive performance based program tailored to local sensitivity AND:




-an education program for SSTS owners on the M&M requirements.





-a program to evaluate the impact of SSTS and advise the LUG P&Z.





-a pgm to determine ‘end of pipe’ lab test requirements to ensure public health.




-establish a program to administer operating permits.




-establish a program to monitor pumpers.




-a reminder program to owners of the annual reporting requirements and of





random system inspections to ensure compliance. 





-an enforcement program with penalties for failure to keep system in compliance.




-a record keeping program of all monitored systems.





-funding program must be established to support the management of the program.


Alternative local standards program to include:




ALS must meet the requirements of MN 115.55 subp 7 A&B AND:





-not apply to Shoreland, Wellhead or Food and beverage areas.





-have criteria for updating non-compliant systems.





-be necessary due to local issues.





-may be less restrictive in areas of low density which make it unreasonable.





-provide procedures, and maps of the ALS area.

7082.0300
Local program administration.


A LUG can request a variance from Chp 7080-81 with explanation as to why it would be 




unreasonable, impractical or not feasible to comply with the code.



A variance to LUG ordinances is allowed.



But no variances to get below the 10,000 gpd mark, or avoid an IPHT. 



LUG must maintain various system records to include NOC, NON, annual list of all tanks 



installed, variance requests, and permit files containing: site evaluation, design, as-built, 



and management plan. 


LUG must enforce code and local ordinances.
7082.0500
Permit program.


LUG’s shall enforce ordinances that regulate septics through local programs to meet the req’s 




of 7080-83.



LUG with an adopted conventional or performance SSTS program, must have a permit program 



that addresses: application requirements, design and soil review procedures, record 



keeping, and reporting to the commissioner.


Permits are required for all new construction and replacements systems.  LUG’s may require 



permits for repairs.



LUG’s with a building code must ensure bedroom additions have a compliant SSTS.



LUG’s must have a procedure to deal with changes to approved/permitted designs.



A qualified employee or licensed inspection business authorized by the LUG must review the



permit application to ensure it meets 7080 standards, and do a soil verification.



An advanced inspector is required for:   ISTS > 2500 gpd,  and MSTS.



Installation must not begin prior to issuance of a permit.



No bedroom permits given unless ISTS is in compliance.  Compliance can be delayed until 



spring for winter additions. 



A licensed inspection business under contract with the LUG must not design or install systems 



that it will also permit or inspect on behalf of the LUG.

7082.0600
System Management.



Management plans are required for Type I-III, to include:




maintenance requirements and frequency,




operating requirements performed by owner, and service provider,



monitoring requirements,




requirement that owner notify LUG when management plan is not being met,




disclosure of the location of the secondary drainfield site,







and any other specific requirements of the LUG.


Operating permits are required for  Type IV-V, and MSTS, to include:



maintenance requirements and frequency,




operating requirements,




monitoring requirements,




compliance limits,




reporting frequency,




requirement that owner notify LUG when operating permit is not being met, with 





corrective actions taken as directed by the LUG,



disclosure of the location of the secondary drainfield site,




and stipulation of acceptable and prohibited discharges.
7082.0700
Inspection program.

LUG’s must have a NEW construction inspection program to enforce 7080 standards on new and replacement septics.


To include:




-Frequency & times of inspection,




-Requirements of the inspection & the protocol. 



A construction compliance inspection for new or replacement systems must be done:




-to ensure compliance with all rules,
-before issuing a bldg permit for a bedroom addition (when a bldg permit pgm exists),



-by an authorized inspector independent of the owner & installer,




-for disclosure purposes when the disclosure is done by someone other than the owner. 


An existing system inspector can subsequently design and install a new system.



An LUG contracted inspector can not design or install a system they would subsequently be 



required to permit and/or do a construction inspection on.


An existing system inspector can inspect a system they designed or installed once it has been 



inspected by an independent party.



An LUG or person contracted by the LUG can not use their position for private business gain.

Subp 3   COC / NOC:


New construction system inspections are given a Certificate of Compliance upon reasonable 



assurance the system was built according to permit specifications. 


LUG’s must develop a COC document or use one by the agency, to include:




- verbage or documentation that the “soil verification” was performed,




-and contain the management plan.

All COC for new construction/replacement must include:  
(LUG inspectors report to include: )   (this is not the  AS-BUILT – see 7083.0020 sub 4))



-owner identification,




-date of inspection,




-system component locations dimensioned or drawn to scale




-well setback distance,




-field check of soil conditions (what does that mean? duplicate of the soil verify?),




-SWF designations as applicable




-class V designations as applicable (injection wells, basic inventory form?)

 



COC’s for new construction must be submitted to the LUG and owner within 15 days and are 



valid for 5 years unless the LUG finds evidence of non-compliance.

Subp 4  EXISTING SYSTEM compliance inspections:



must verify that:




1) No IPHT exists.





a) no surfacing effluent





b) no re-occurring back-up into the home.





c) no electrical hazards





d) no weak riser lids





e) no “other” issues as deemed by inspector.

2) Protective of groundwater:

a) Tanks do not leak    





b) Adequate vertical separation 




c) all unused systems are abandoned.





d) no “other” issues as deemed by inspector.




3) Operating permit is being met.   
Must use MPCA existing system inspection form, and include any locally req’d forms.  

Previous tank integrity assessments  <3yrs old may be used. 



Tank integrity portion can be performed by any SSTS professional.



COC/NON must be signed off by an “inspector”, even for just holding tanks.

Previous soil verifications may also be used.

Soil verify portion must be performed by an inspector by using:



- New soil borings,




- or by using 2 previous independent verifications, (design or previous inspections)





Once 2 verify’s have occurred, no further verifications are necessary.

IPHT issues to be performed by an inspector.
Operating permit (type IV & V & MSTS) and nitrogen req’s to be performed by an Intermediate or Advanced Inspector or Service Provider.



Existing system compliance inspections must be submitted to the LUG and owner within 15 days 


and are good for 3 years unless evidence is found to the contrary,


Subp 5     Soil verify disagreement procedures for design or compliance inspections: 

Dispute between two designers/inspectors:



1) Follow local procedures



2) Disputing parties must meet at the site and attempt to resolve their differences.  


3) If still in disagreement:




a) get LUG opinion




b) get opinion from SSTS technical evaluation committee (if there is one)




c) if no a) or b), then must get opinion from a Professional Soil Scientist who 
    is an SSTS professional.



4) If still in dispute,  LUG shall consider all previous information and make a final call.

Dispute between LUG & designer/inspector: (same as above but PSS is optional)


1) Follow local procedures



2) Disputing parties must meet at the site and attempt to resolve their differences.  



3) If still in disagreement:




b) get opinion from SSTS technical evaluation committee (if there is one)




c) get opinion from a Professional Soil Scientist who is an SSTS professional.



4) If opinions from 3) are not sought or still in dispute, 

 LUG shall consider all previous information and make a final call.

NOTE:

AS-BUILTS are required per 7083.0760



And defined in 7083.0020 subp 4 as
drawings and documentation specifying the final: 

· Location, elevation, size and type of components.

· Results of materials testing (what is this?)

· Conditions during construction (what is this?)

· Include a certified statement (what is a certified statement - signature?)

